Kiosk Computer Specifications

| System Hardware Specifications | Eon |
| --- | --- |
| Motherboard | 5BLMP |
| CPU | Integrated National Semiconductor Geode microprocessor (GXmLV 233 MHz) |
| Chipset | Cx5530 |
| BIOS | Award 256K byte (2Mbit) |
| Memory | 1x SODIMM / 1x DIMM 32-128MB Shipped in basic configuration with 32MB RAM |
| I/O | NS3351 |

| Video | Eon |
| --- | --- |
| VGA Chip | Geode GX VSA |
| Monitor output resolutions supported | 640x480, 800x600, 1024x768, 1152x864, 1280x1024 |
| Color depth | 16-bit color to 1024x768, 8-bit color to 1280x1024 |

| Storage Options | Eon |
| --- | --- |
| Hard disk | 3.5"/2.5" (optional) |
| CD-ROM | 20x/24x CD-ROM (optional) |
| Floppy | 1.44 MB floppy (optional) |
| Flash Disk | Disk On Chip 2000 |

| Network | Eon |
| --- | --- |
| LAN | Realtek 8139A |
| LAN Speed | 10 / 100 Mbits / second |
| Floppy / Smart Card | 1.44 MB floppy (optional) |
| Flash Disk | Disk On Chip 2000 (shipped with 16 MB DiskOnChip installed) |

| Audio | Eon |
| --- | --- |
| Chip | VSA, Sound Blaster Compatible |
| Input/output | Stereo Line in(CD in), Line out, Mic |

| I/O | Eon |
| --- | --- |
| LAN | One RJ45 connector - autosense 10BaseT / 100BaseT twisted-pair Ethernet |

| Ports | Eon |
| --- | --- |
| CRT | One D-sub 15-pin female |
| Audio | Stereo Line in, Line out, Microphone |
| Keyboard | One PS/2 type connector |
| Mouse | One PS/2 type connector |
| Serial | Two D-sub 9-pin male |
| Parallel | One D-sub 25-pin female |
| USB | 2 (Type A) |
| Power Connector | AC Inlet 60w universal |
| Expansion Slots | 1x ISA/PCI shared |

| Case | Eon |
| --- | --- |
| Dimensions | 290(W)mm x 220(D)mm x 53(H)mm -- in horizontal orientation |