Alkindi Kiosk Connectivity Options

July 13, 2000

Synopsis

## Kiosk Connectivity

Many options present themselves when considering connectivity between Alkindi consumer kiosks and the central information store operated by Alkindi.  All Internet solutions should include security through encryption via a VPN.  The operating system on the kiosk (least expensive) or the routers connecting the kiosk to the Internet (most expensive) will handle the VPN.  Alternatively, a VPN software solution could be such as CheckPoint’s SecuRemote client software and firewall.   However this would limit the choices of operating systems on the kiosk.

When a set number of kiosks have been deployed it may be necessary to switch from an inexpensive connectivity solution that is managed internally to a more expensive solution that is managed by an ISP.  Costs, overhead of network management, number of kiosks, and anticipated growth, among other factors, would determine this.

## Connectivity Description

# Leased Line

56K and up

No VPN or other security equipment is needed.

The line is a point to point connection between each terminal and Alkindi.  The line cannot be compromised either by eavesdropping or other means.  The machine is off the Internet and as such cannot be attacked, brought down, nor can a denial of service be accomplished.  This is also the cheapest solution, in terms of pure connectivity alone (not security), other than SDSL.  If the line goes down, it cannot reroute as if it were an Internet connection.  There may be some delays in getting back online but the ISP would be responsible and SLAs would cover such outages.  Over time, the overhead in maintaining one’s own Network Operations Center may become onerous.  At some point, it may be advisable to switch to a true Internet connection provided by an ISP (not a DSL line).

# UUNet 56K Internet Line

56K bandwidth line to Burstable T1

Routing Equipment

The terminal would handle securing Internet access and VPN connectivity.  UUnet is responsible for the connection from end to end.  SLA would have uptime guarantees and customers would have a more dependable connection.  The terminal is responsible for VPN via IPSEC and its own security.

# UUSecure Solution

56K bandwidth line to Burstable T1

Routing Equipment (VPN capable router)

They turn the routing tables down so there is no Internet connectivity and just a point to point connection.  This is in addition to a VPN connection via IPSEC.  UUnet is responsible for the connection from end to end.  SLA would have uptime guarantees and customers would have a more dependable connection.  This solution incurs less processing toll on the terminal as it is offloaded onto the routing equipment, for the most part this is a trivial concern as the flow of data is not that large.

# SDSL

384K to 768K

Equipment included

Inherent is the inability to resolve issues in a timely manner.  Bell Atlantic is ultimately responsible for the line so downtime can last for days or weeks.  Customer service might suffer.  SLAs with the provider (UUnet) are only good for specific pieces of equipment and the DSL provider (Covad and Bell Atlantic) cannot be held responsible for unacceptable amounts of downtime.  The terminal via IPSEC VPN would handle security.

# Cable Modems

500K to ~1Mb

Equipment included

Cable modem connections are not available in all areas, with entire zip codes not having service available. Resorting to another type of connection might be necessary – this would lead to a more heterogeneous networking environment, multiple vendors, and increased overhead.  The Service Level Agreements are lacking in bandwidth and uptime guarantees.  Cable modems do offer fast access and quick installs but since it is not available in most areas and the bandwidth is ‘shared’ service may suffer as a result.

## Connectivity Recommendation

DSL has the best price/performance ratio and migrating to a managed solution in the future would be made easier as it more closely mirrors DSL in its form and function.

For the first kiosk implementations, DSL is considered best. After a certain number have been deployed, leased line connection may be advisable to reduce management overhead. Both solutions are low cost connectivity solutions.  DSL is more flexible in its routing but is more intensive when it comes to configuration (IPSEC VPN and securing the kiosk) of the kiosk.  DSL also falls under the jurisdiction of Bell Atlantic and their slow response times and the reseller does not guarantee uptime.  Leased lines are inherently far more secure and are dedicated digital connections, unlike DSL or any other internet connectivity option.  With superior uptime and better response times in case of failure, leased lines are preferred for high availability scenarios.

## Kiosk Architecture

Security is of high priority, as customer experience will be severely affected by downtime.   Denial of service attacks, crackers, script kiddies, and others can bring down a kiosk.  Rollout and remote management of the kiosks is a critical factor.  Extra hardware can be employed to protect the kiosk or the operating system can be hardened.  Operating system robustness relates to uptime and ease of administration.

- Security

- Rollout / Cloning

- Remote Administration

- Robustness

- User Experience

Unix has proved itself in all the listed categories.  In particular OpenBSD is the most secure operating known today and supports the latest in security protocols.  It is also free of fees and alterations to the system do not have to be made public.  It can also seamlessly run Linux binaries without latency.  Linux has been deployed in a variety of ways including Point Of Sale (POS) systems across the country (Burlington Coat Factory Stores).  The interface (Linux/OpenBSD) is very customizable and locking down the system from a local security standpoint is not trivial but it is not onerous either.  Linux does have the caveat that, if modified, any changes in code must be released to the open source community.  This has led companies to embrace the BSD operating systems when it comes to creating proprietary solutions.  Unix has proven track records in uptime and remote administration tasks.  The number of people required to administer a Unix network is approximately ¼ that of non-Unix operating systems.

## Next Steps

Implementation decisions need to be addressed more closely and follow defined requirements.  Then further hands-on research can be performed on kiosk architecture.

Pricing

| Leased Line Connections (MCI) (~2 mile distance) | Monthly Port Cost | Monthly Local Loop | Monthly total | Setup cost | Setup Router (VPN) | Setup Cost Total |
| --- | --- | --- | --- | --- | --- | --- |
| 56K |  |  | 275 |  |  |  |
| 112K - 1.288Mbps |  |  | 580 |  |  |  |
| Internet Connection (UUNet) |  |  |  |  |  |  |
| 56K | 595 | 150 | 745 | 100 | 595 | 695 |
| 768K | 768 | 200 | 968 | 1500 | 1500 | 3000 |
| UUSecure Solution (UUNet) |  |  |  |  |  |  |
| 56K | 800 | 150 | 950 | 100 | 595 | 695 |
| Burstable T1 | 1500 | 200 | 1700 | 1500 | 1500 | 3000 |
| SDSL (UUNet) |  |  |  |  |  |  |
| 384K | 200 |  | 200 | 200 | 450 | 650 |
| ADSL (Red Connect) |  |  |  |  |  |  |
| 320kb/320kb | 129.95 |  | 129.95 |  |  |  |
| 640kb/320kb | 159.95 |  | 159.95 |  |  |  |
| All contracts are yearly |  |  |  |  |  |  |

## Contact Information

## Red Connect

1-800-RED-9001

http://www.redconnect.com/

## UUNet

Jim Poulos

703-886-6026

http://www.uu.net/

## MCI/WorldCom Leased Lines

Tom Kasakria

914-312-6249