Production Network

July 27, 2000

Overview

This document outlines the production network requirements for Alkindi’s network environment, containing web, application and database services, kiosk connectivity and general administrative functions such as backups, VPNs etc.

## Redundancy

Redundancy is highly important to ensure that the system is up and running 99% of the time. Wherever possible, redundancy has been built in by adding secondary failover units, or load balancing / failover systems. All single access points (load balancer, firewall, switches) have been clustered. Machines are equipped with dual network cards, a cheap and efficient way to ensure that uptime is maximized.

## Firewalls

Alkindi user data is highly confidential and a breach in security must be prevented as much as possible. The network diagram illustrates the use of 3 sets of firewalls to shield the user data from intrusion. Firewalls are fully redundant using standard failover, and are implemented by at least two different vendors to make the system more difficult to hack into. Firewalls include Cisco Pix, Nokia Firewalls and Checkpoint on Sun.

Firewalls are configured to only allow traffic to and from their designated partners. The strictest rules (port restrictions, IP restrictions, stateful inspection) will be implemented to counter common attacks such as buffer overruns, ip spoofing and port scanning.

## Load Balancing

Load balancers should support IP, cookie and URL based load balancing, such as Arrowpoint’s CS100 and CS800 or F5 BigIP. Web servers, and potentially application tier servers will be load balanced according to certain criteria, depending on the architecture of the system. Most likely the system will use transparent, cookie-based load-balancing, which allows for independent development of web and application services. Users will always hit the same machine they connected, provided they are still in the same session. Disadvantages are that the user requires cookies (the site will most likely be using cookies for other purposes so this factor is most likely moot),  and that a lower degree of load-balancing is achieved since it could occur that many users hitting Web Server A all take 20 minutes to perform their transactions while all the users hitting Web Server B require only 5 seconds. This can result in one server being overloaded while others are idle. This will not be significant for the first years of operation, and the cost of using back-end, i.e. application aware, load balancing will be prohibitive.

Requirement for year 3 is that the load balancer support geographic load-balancing since Alkindi will begin operating a secondary site, most likely on the opposite coast. Both F5 and Arrowpoint support this.

## Database

The database is the central storage unit and is not easily replicated. To ensure high availability, the database has been clustered, using an external RAID and two database computers to deliver data. In the event that one server fails, the other will automatically take over control. The particular implementation is TBD once the software spec is written. It may be simpler to perform lazy replication to an offline database that can be brought online in a matter of minutes if the primary server were to fail. Options include Oracle Parallel Server, Veritas Replication for Oracle among others.

User data may also be distributed on different database servers to ensure that the whole system is not compromised once a breach occurs.

## Switched Network

The network is a 100Mb switched environment, all servers have dedicated 100MB ports. Switches are stacked using fiber backbones, and high throughput servers may use full-duplex (200MB) connections if required. Cisco switches since they offer high performance and easy administration.

## Backups

Backups will be performed using a tape storage unit, TBD depending on hardware architecture.

## Remote Access

Remote Access from a portion of the Alkindi office (the production systems) will be allowed using a VPN connection from the office to the lowest level firewall in the production system.

## Kiosk Connectivity

While still TBD, kiosks will most likely connect using DSL to the internet, and will therefore require additional security. SSL using dual certificates will be used to ensure private and authenticated communication. The hosting facility will not require special or additional equipment in this case.

Backup connectivity may be initiated using either dial-up internet access or direct RAS connectivity to the hosting facility. TBD.

Version 1 (November 1st)

To ensure timely implementation and reduced cost, it is recommended that the following setup be used:

1 Checkpoint Firewall	$13,000

1 Cisco Pix	$15,000

1 Load Balancer CS100	$10,000

2 Web Servers	$12,000

1 Database Server	$30,000

1 OLAP Server	$6,000

2 App Tier Servers	$12,000

switches/misc network equip	$10,000

Total:	$108,000

This eliminates much of the redundancy and creates several single points of failure.

Production system IP’s

| ALK-WWW-01: |  | 192.168.1.20/192.168.128.20 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ALK-WWW-02: |  | 192.168.1.21/192.168.128.21 |  |  |  |  |
| ALK-KWS-01: |  | 192.168.1.22/192.168.128.22 |  |  |  |  |
| ALK-KWS-02: |  | 192.168.1.23/192.168.128.23 |  |  |  |  |
| ALK-APP-01: |  | 10.10.1.51 |  |  |  |  |
| ALK-APP-02: |  | 10.10.1.52 |  |  |  |  |
| ALK-APP-02: |  | 10.10.1.53 |  |  |  |  |
| ALK-OLAP-01: |  | 10.10.1.30 |  |  |  |  |
| ALK-OLAP-02: |  | 10.10.1.31 |  |  |  |  |
| ALK-DB-01: |  | 10.10.1.20 |  |  |  |  |
| ALK-DB-02: |  | 10.10.1.21 |  |  |  |  |
| ALK-SW-01/02: |  |  |  |  |  |  |
| ALK-SW-03/04: |  |  |  |  |  |  |
| ALK-SW-05/06: |  |  |  |  |  |  |
| ALK-SW-07/08: |  |  |  |  |  |  |
| ALK-SW-09/10: |  |  |  |  |  |  |
| ALK-FW-01: |  |  |  |  |  |  |
| ALK-FW-02: |  |  |  |  |  |  |
| ALK-FW-03: |  |  |  |  |  |  |
| ALK-FW-04: |  |  |  |  |  |  |
| ALK-FW-05: |  |  |  |  |  |  |
| ALK-FW-06: |  |  |  |  |  |  |
| ALK-LB-01: |  |  |  |  |  |  |
| ALK-ADMIN-01: |  | 10.10.1.10 |  |  |  |  |
| ALK-CS-01: |  | 10.10.1.11 |  |  |  |  |
| ALK-BCK-01: |  | 10.10.2.12 |  |  |  |  |
| Switch: |  |  |  |  |  |  |
| SW01-SW10 reside on Cisco Catalyst 3524 |  |  |  |  |  |  |
| Firewalls 1 and 2 are Nokia firewalls |  |  |  |  |  |  |
| Firewalls 3 and 4 are Checkpoint |  |  |  |  |  |  |
| Firewalls 3 and 4 are Cisco Pix |  |  |  |  |  |  |
| Load Balancer is an ArrowPoint CS800 LB1, LB2, LB3 reside on this machine |  |  |  |  |  |  |